# Lab Report 2

Zones, Interfaces, Address Objects & Security Policy Configuration

Student: Ahmad

Platform: PNETLab (virtual lab environment)

Device Under Test: Palo Alto Networks VM-Series Firewall (PA-PRI)

Date: September 2026

# 1. Objective

Configure the three security zones, their interfaces, the required address objects, and the full eight-rule security policy on the primary firewall, then verify every rule matches its intended traffic using the CLI rather than visual inspection alone.

# 2. Zone and Interface Configuration

| Zone | Interface | Subnet | Purpose |
| --- | --- | --- | --- |
| Untrust | ethernet1/1 | 172.16.203.0/24 | Simulated internet / outside world |
| Trust | ethernet1/4 | 192.168.10.0/24 | Internal LAN clients |
| DMZ | ethernet1/5 | 172.16.20.0/24 | Publicly reachable web server |

Address objects created to keep the policy readable and reusable:

- LAN-Subnet = 192.168.10.0/24

- Web-Server = 172.16.20.10/32

# 3. Security Policy

Built specific-to-general, with explicit deny rules placed above the implicit deny-all PAN-OS always applies last — the firewall evaluates top-down and stops at the first match, so a broad rule above a narrow one would silently swallow the traffic the narrow rule exists to catch.

| # | Rule Name | From → To | Destination | App/Service | Action |
| --- | --- | --- | --- | --- | --- |
| 1 | Allow-LAN-to-Web | Trust→Untrust | any | web-browsing, ssl | Allow |
| 2 | Allow-LAN-to-DNS | Trust→Untrust | any | dns | Allow |
| 3 | Allow-LAN-to-Ping-Internet | Trust→Untrust | any | ping | Allow |
| 4 | Allow-Internet-to-WebServer | Untrust→DMZ | Web-Server | web-browsing, ssl | Allow |
| 5 | Allow-LAN-to-WebServer-HTTPS | Trust→DMZ | Web-Server | ssl | Allow |
| 6 | Allow-LAN-to-WebServer-Ping | Trust→DMZ | Web-Server | ping | Allow |
| 7 | Deny-DMZ-to-LAN | DMZ→Trust | any | any | Deny |
| 8 | Deny-Internet-to-LAN | Untrust→Trust | any | any | Deny |

Rule 7 is the core security boundary of the design: if the DMZ web server is ever compromised, this rule stops an attacker from pivoting into the internal LAN. Rules 7 and 8 are written explicitly, rather than relying on the implicit deny-all, because the implicit rule does not log by default — explicit deny rules give both visibility and a human-readable statement of intent. Logging (Log at Session End) was enabled on every rule.

# 4. Verification Methodology

Each rule was verified individually using the CLI test command, which simulates a given flow against the live policy and reports which rule it matches:

test security-policy-match from <zone> to <zone> source <ip> destination <ip> destination-port <port> protocol <proto>

A gotcha was identified and documented during this step: omitting the from/to zone parameters allows the command to auto-derive the destination zone, and this auto-derivation returned an incorrect match on one test (Rule 5) even though the underlying interface, routing, and zone configuration were independently confirmed correct via show routing fib-lookup and the routing table. Re-running the identical test with from/to specified explicitly returned the correct match. From that point forward, from/to zones were always specified explicitly.

## 4.1 Verification Results

| Test Flow | Expected Rule | Result |
| --- | --- | --- |
| Trust → Untrust, TCP/443 (HTTPS) | Rule 1 | Match confirmed |
| Trust → Untrust, UDP/53 (DNS) | Rule 2 | Match confirmed |
| Trust → Untrust, ICMP | Rule 3 | Match confirmed |
| Untrust → DMZ, TCP/443 to Web-Server | Rule 4 | Match confirmed |
| Trust → DMZ, TCP/443 to Web-Server | Rule 5 | Match confirmed (after from/to fix) |
| Trust → DMZ, ICMP to Web-Server | Rule 6 | Match confirmed |
| DMZ → Trust, TCP/80 | Rule 7 | Match confirmed (denied) |
| Untrust → Trust, TCP/80 | Rule 8 | Match confirmed (denied) |

All eight rules were confirmed to match exactly the traffic they were designed for, with no shadowing between rules. The “Highlight Unused Rules” indicator was also reviewed in the GUI; at this stage all rules show as unused, which is expected since no live traffic had yet been generated — this check becomes meaningful once end-to-end traffic testing is performed.

# 5. Evidence

- Screenshot: Policies > Security rule list showing all 8 rules in order, with logging enabled

- Screenshot: CLI output of each test security-policy-match command and its matched rule

- Screenshot: the from/to zone ambiguity and its corrected re-test, documented as a troubleshooting finding

# 6. Conclusion

The firewall's zone structure, address objects, and complete 8-rule security policy are configured and individually verified against the design intent via CLI. This policy is the configuration that Lab Report 3's High Availability pairing is built to replicate onto the secondary firewall.