# Lab Report 3

High Availability Deployment & Configuration Synchronization

Student: Ahmad

Platform: PNETLab (virtual lab environment)

Devices Under Test: PA-PRI (Active) and PA-SEC (Passive) — Active/Passive HA pair

Date: September–October 2026

# 1. Objective

Deploy a second firewall (PA-SEC), pair it with PA-PRI in Active/Passive High Availability, and verify that configuration synchronization correctly replicates the zones, objects, and security policy from the Active peer — removing the need to manually rebuild the policy a second time.

# 2. HA Link Design

| Link | Interface | Subnet | Purpose |
| --- | --- | --- | --- |
| HA1 (Control) | ethernet1/2 | 10.0.1.0/30 | Heartbeats and configuration sync |
| HA2 (Data) | ethernet1/3 | 10.0.2.0/30 | Session-state synchronization |

Both links are direct point-to-point cables between PA-PRI and PA-SEC — no intermediate switching, no gateway required.

# 3. Procedure

## 3.1 PA-SEC Deployment

PA-SEC was built on the same PAN-OS 11.x image and resource profile established in Lab Report 1 (4 vCPU / 8192 MB / 6 interfaces), to avoid the boot failure already diagnosed and resolved there.

Minimal CLI configuration was applied directly to reach a working state: management/admin-access IP, Untrust zone/interface, and default route — mirroring the same data-interface management technique from Lab Report 1, using a unique address (172.16.203.57) to allow independent access to PA-SEC prior to HA pairing.

## 3.2 HA Configuration

| Setting | PA-PRI | PA-SEC |
| --- | --- | --- |
| Group ID | 1 | 1 |
| Mode | Active-Passive | Active-Passive |
| HA1 (Control Link) | ethernet1/2, 10.0.1.1/30 | ethernet1/2, 10.0.1.2/30 |
| HA2 (Data Link) | ethernet1/3, 10.0.2.1/30 | ethernet1/3, 10.0.2.2/30 |
| Device Priority | 90 (preferred Active) | 110 |
| Peer HA1 IP | 10.0.1.2 | 10.0.1.1 |
| Config Sync | Enabled | Enabled |

## 3.3 Configuration Synchronization

With Config Sync enabled, the Active peer automatically pushes zones, interfaces, address objects, and the full security policy to the Passive peer once the HA session establishes — PA-SEC's policy did not need to be manually rebuilt. Only device-local settings (management IP, HA link IPs, Device Priority, hostname) require independent configuration on each firewall.

Synchronization was confirmed through three independent observations:

- PA-SEC's ethernet1/1 IP changed from its originally assigned unique address (172.16.203.57) to match PA-PRI's (172.16.203.56) — confirming interface configuration was pushed from the Active peer, standard behavior for a shared-segment HA pair.

- show running security-policy on PA-SEC returned all 8 security rules, though only 4 had been manually entered there.

- show zone on PA-SEC listed the DMZ zone, which had never been manually created on that device.

# 4. Issues Encountered and Resolutions

| Issue | Root Cause | Resolution |
| --- | --- | --- |
| HA state showed “not synchronized” despite HA1 connectivity being up | Device Dictionary (IoT Content) version mismatch between the two PAN-OS images (one had version 1-211 installed, the other none) | Compared show system info output side-by-side to isolate the single differing field; manually triggered request high-availability sync-to-remote running-config, which succeeded — the mismatch proved advisory, not a hard blocker |
| PA-SEC's independent GUI access stopped working after sync | Expected consequence of interface config sync overwriting PA-SEC's unique admin-access IP to match PA-PRI's | Documented as correct production HA behavior rather than a fault; PA-SEC is now managed via console, or would be reachable at that shared address only if it becomes Active |
| DMZ zone/interface config present on PA-SEC but non-functional | ethernet1/5 was not yet physically connected to the DMZ switch in the topology | Wired the link and brought up the corresponding switch port (bridge-group membership) to match |

# 5. Evidence

- Screenshot: show high-availability state on PA-PRI (state: active, priority 90) and PA-SEC (state: passive, priority 110), peer connection status: up

- Screenshot: show system info comparison showing the isolated device-dictionary-version mismatch

- Screenshot: manual sync command output (“HA synchronization job has been queued on peer”)

- Screenshot: show interface ethernet1/1 on PA-SEC confirming the IP changed to match PA-PRI post-sync

- Screenshot: show running security-policy and show zone on PA-SEC confirming all 8 rules and the DMZ zone were received via sync

# 6. Conclusion

PA-PRI and PA-SEC form a functioning Active/Passive HA pair: HA1/HA2 links are established, role election behaves as configured (PA-PRI Active, PA-SEC Passive), and Configuration Synchronization was independently verified to replicate zones, address objects, and the full security policy from the Active peer to the Passive one. This completes the high-availability objective of the lab, building directly on the firewall deployment from Lab Report 1 and the security policy from Lab Report 2.