Final Network Implementation Report

Multi-Site Enterprise Network Design

VLSM  •  VLANs  •  Trunking  •  EtherChannel  •  Rapid PVST+  •  Router-on-a-Stick  •  Static Routing

# 1. Introduction & Lab Requirements

### Objective

Design and implement a scalable, resilient, multi-site enterprise network using VLSM, VLANs, Trunking, EtherChannel, Rapid PVST+, Router-on-a-Stick, and Static Routing.

### Topology Overview

- 3 Core Routers (Cisco 2911): R1, R2, R3.

- 6 Layer 2 Switches: S1 through S6.

- 11 End-User PCs: Distributed across 7 distinct subnets.

- Physical Links: Includes a 3-link EtherChannel bundle (S6-S3) and a redundant triangle topology (S5-S1-S2) with load balancing.

### Requirements Met

- VLSM applied to 83.217.64.0/20 to minimize IP waste.

- VLANs segmented by department/location.

- Trunking (802.1Q) for inter-switch and router connections.

- Router-on-a-Stick for inter-VLAN routing.

- EtherChannel (LACP) for redundancy and load balancing.

- Rapid PVST+ for loop prevention and traffic path optimization.

- SVIs for remote switch management.

- Static Routing for full end-to-end connectivity.

# 2. VLSM Calculation Summary

We subnetted 83.217.64.0/20 (4096 total addresses) into 9 subnets.

### Calculation Rule

2ⁿ − 2 ≥ Required Hosts

- User Subnets (A-G): Required 4 usable IPs for future expansion. 2³ − 2 = 6 ≥ 4. Mask = /29 (Block size = 8).

- WAN Links (H-I): Required 2 usable IPs. 2² − 2 = 2. Mask = /30 (Block size = 4).

- Management (VLAN 99): Used /28 subnets (Block size = 16) to accommodate multiple switches per region.

# 3. Full IP Addressing & Port Reference Table

This table serves as the master reference for all device configurations and testing.

| Device | Interface / VLAN | IP Address | Subnet Mask | Default Gateway | Connected To / Purpose |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/1.10 | 83.217.64.1 | 255.255.255.248 | N/A | Gateway VLAN 10 (A) |
| R1 | G0/1.20 | 83.217.64.9 | 255.255.255.248 | N/A | Gateway VLAN 20 (B) |
| R1 | G0/1.30 | 83.217.64.17 | 255.255.255.248 | N/A | Gateway VLAN 30 (C) |
| R1 | G0/1.99 | 83.217.64.97 | 255.255.255.240 | N/A | Gateway VLAN 99 (Left Mgmt) |
| R1 | G0/0 | 83.217.64.57 | 255.255.255.252 | N/A | WAN Link to R3 (.58) |
| R3 | G0/2.50 | 83.217.64.33 | 255.255.255.248 | N/A | Gateway VLAN 50 (E) |
| R3 | G0/2.60 | 83.217.64.41 | 255.255.255.248 | N/A | Gateway VLAN 60 (F) |
| R3 | G0/2.70 | 83.217.64.49 | 255.255.255.248 | N/A | Gateway VLAN 70 (G) |
| R3 | G0/2.99 | 83.217.64.129 | 255.255.255.240 | N/A | Gateway VLAN 99 (Middle Mgmt) |
| R3 | G0/0 | 83.217.64.58 | 255.255.255.252 | N/A | WAN Link to R1 (.57) |
| R3 | G0/1 | 83.217.64.61 | 255.255.255.252 | N/A | WAN Link to R2 (.62) |
| R2 | G0/0.40 | 83.217.64.25 | 255.255.255.248 | N/A | Gateway VLAN 40 (D) |
| R2 | G0/0.99 | 83.217.64.113 | 255.255.255.240 | N/A | Gateway VLAN 99 (Right Mgmt) |
| R2 | G0/1 | 83.217.64.62 | 255.255.255.252 | N/A | WAN Link to R3 (.61) |
| S5 | VLAN 99 SVI | 83.217.64.98 | 255.255.255.240 | 83.217.64.97 | Management (Transit Switch) |
| S1 | VLAN 99 SVI | 83.217.64.99 | 255.255.255.240 | 83.217.64.97 | Management (Left Lower) |
| S2 | VLAN 99 SVI | 83.217.64.100 | 255.255.255.240 | 83.217.64.97 | Management (Right Lower) |
| S4 | VLAN 99 SVI | 83.217.64.130 | 255.255.255.240 | 83.217.64.129 | Management (Middle) |
| S6 | VLAN 99 SVI | 83.217.64.114 | 255.255.255.240 | 83.217.64.113 | Management (Right Transit) |
| S3 | VLAN 99 SVI | 83.217.64.115 | 255.255.255.240 | 83.217.64.113 | Management (Bottom Right) |
| PC A1 | Fa0 (S1 Fa0/2) | 83.217.64.2 | 255.255.255.248 | 83.217.64.1 | VLAN 10 |
| PC A2 | Fa0 (S2 Fa0/1) | 83.217.64.3 | 255.255.255.248 | 83.217.64.1 | VLAN 10 |
| PC B1 | Fa0 (S1 Fa0/3) | 83.217.64.10 | 255.255.255.248 | 83.217.64.9 | VLAN 20 |
| PC B2 | Fa0 (S2 Fa0/2) | 83.217.64.11 | 255.255.255.248 | 83.217.64.9 | VLAN 20 |
| PC C1 | Fa0 (S1 Fa0/4) | 83.217.64.18 | 255.255.255.248 | 83.217.64.17 | VLAN 30 |
| PC C2 | Fa0 (S2 Fa0/3) | 83.217.64.19 | 255.255.255.248 | 83.217.64.17 | VLAN 30 |
| PC D1 | Fa0 (S3 Fa0/1) | 83.217.64.26 | 255.255.255.248 | 83.217.64.25 | VLAN 40 |
| PC D2 | Fa0 (S3 Fa0/2) | 83.217.64.27 | 255.255.255.248 | 83.217.64.25 | VLAN 40 |
| PC E1 | Fa0 (S4 Fa0/4) | 83.217.64.34 | 255.255.255.248 | 83.217.64.33 | VLAN 50 |
| PC F1 | Fa0 (S4 Fa0/2) | 83.217.64.42 | 255.255.255.248 | 83.217.64.41 | VLAN 60 |
| PC G1 | Fa0 (S4 Fa0/1) | 83.217.64.50 | 255.255.255.248 | 83.217.64.49 | VLAN 70 |

# 4. Implementation Steps (Phases 1-8)

### Phase 1: VLAN Database Creation

Action: Created VLANs on each switch (10, 20, 30 on S5/S1/S2; 50, 60, 70 on S4; 40 on S6/S3; and VLAN 99 on all).

Design Choice: Dedicated VLAN 99 for management to segregate administrative traffic from user data, enhancing security.

### Phase 2: Access Port Assignment

Action: Set ports connected to PCs to switchport mode access and assigned them to their respective VLANs (e.g., S1 Fa0/2 in VLAN 10).

Design Choice: Ensures end-devices are properly segmented into their broadcast domains.

### Phase 3: Trunk Configuration & VLAN Pruning

Action: Configured all inter-switch and router-switch links as 802.1Q trunks. Restricted allowed VLANs on each trunk (e.g., S5 only allows 10,20,30,99; S6 only allows 40,99).

Design Choice: VLAN Pruning enhances security and reduces unnecessary broadcast traffic crossing links.

### Phase 4: Router-on-a-Stick (Inter-VLAN Routing)

Action: Created sub-interfaces on R1, R2, and R3 with encapsulation dot1Q.

Design Choice: Allows a single physical router interface to route traffic for multiple VLANs, eliminating the need for multiple physical links.

### Phase 5: EtherChannel (S6 ↔ S3)

Action: Bundled three physical links (Fa0/17-19) into Port-Channel 1 using LACP (Active mode) and configured it as a trunk for VLANs 40 and 99.

Design Choice: Provides redundancy (fault tolerance) and load balancing (increased bandwidth) across the 3 cables.

### Phase 6: SVIs & Management Subnet Split

Action: Assigned SVI IPs to all switches in VLAN 99. Due to routing conflicts (Connected routes overriding Static routes), we split the management subnet into three separate /28 ranges: Left (.96/28), Middle (.128/28), and Right (.112/28).

Design Choice: Ensures routers do not incorrectly ARP for remote management IPs on their local interfaces, guaranteeing reliable remote access.

### Phase 7: Rapid PVST+ Path Control (Load Balancing)

Action: Set S5 as the Root Bridge. Adjusted spanning-tree costs to force specific paths:

• VLAN 10 & 20 (A1/B1): S1 → S2 → S5 (Blocked S1 Fa0/1, adjusted S1 priority to 40960 to fix MAC address tie-breaking for VLAN 20).

• VLAN 30 (C2): S2 → S1 → S5 (Blocked S2 Fa0/5).

Design Choice: Optimizes traffic flow by utilizing redundant links rather than keeping them in a blocking state, while maintaining loop prevention.

### Phase 8: Static Routing (End-to-End Connectivity)

Action: Added static routes on all routers:

• R1 routes to 40, 50, 60, 70, 112, 128 via R3 (.58).

• R3 routes to 10, 20, 30, 96 via R1 (.57), and to 40, 112 via R2 (.62).

• R2 routes to 10, 20, 30, 50, 60, 70, 96, 128 via R3 (.61).

Design Choice: Enables full reachability between all subnets without relying on dynamic routing protocols, keeping the design simple and deterministic.

# 5. Network Connectivity Testing Table

Perform the following tests from the Source PC Command Prompt (ping <IP>). Mark Pass or Fail in the final column.

| # | Source PC (IP) | Destination Device | Destination IP | Expected Result | Actual Result |
| --- | --- | --- | --- | --- | --- |
| 1 | A1 (83.217.64.2) | Default Gateway (R1) | 83.217.64.1 | Reply (TTL=255) | PASS |
| 2 | A1 (83.217.64.2) | S5 (Left Mgmt) | 83.217.64.98 | Reply (TTL=255) | PASS |
| 3 | A1 (83.217.64.2) | S4 (Middle Mgmt) | 83.217.64.130 | Reply (TTL=253) | PASS |
| 4 | A1 (83.217.64.2) | S3 (Right Mgmt) | 83.217.64.115 | Reply (TTL=252) | PASS |
| 5 | A1 (83.217.64.2) | D1 (VLAN 40 PC) | 83.217.64.26 | Reply (TTL=252) | PASS |
| 6 | B1 (83.217.64.10) | C2 (VLAN 30 PC) | 83.217.64.19 | Reply (TTL=254) | PASS |
| 7 | D1 (83.217.64.26) | Default Gateway (R2) | 83.217.64.25 | Reply (TTL=255) | PASS |
| 8 | D1 (83.217.64.26) | S6 (Right Mgmt) | 83.217.64.114 | Reply (TTL=255) | PASS |
| 9 | D1 (83.217.64.26) | S5 (Left Mgmt) | 83.217.64.98 | Reply (TTL=253) | PASS |
| 10 | D1 (83.217.64.26) | A1 (VLAN 10 PC) | 83.217.64.2 | Reply (TTL=253) | PASS |
| 11 | E1 (83.217.64.34) | R3 WAN IP (to R2) | 83.217.64.61 | Reply (TTL=255) | PASS |
| 12 | G1 (83.217.64.50) | R1 WAN IP (to R3) | 83.217.64.57 | Reply (TTL=254) | PASS |

# Appendix: Full Configuration Commands

This appendix contains the complete CLI configuration for every phase of implementation, including troubleshooting fixes applied during testing.

## Phase 1: VLAN Database Creation

We created the local VLANs on each switch so they could forward traffic for those specific subnets.

On S5, S1, S2:

vlan 10

name Subnet_A

exit

vlan 20

name Subnet_B

exit

vlan 30

name Subnet_C

exit

vlan 99

name Management

exit

On S4:

vlan 50

name Subnet_E

exit

vlan 60

name Subnet_F

exit

vlan 70

name Subnet_G

exit

vlan 99

name Management

exit

On S6, S3:

vlan 40

name Subnet_D

exit

vlan 99

name Management

exit

## Phase 2: Access Ports (PC Assignments)

We set the switch ports connected to PCs to "Access" mode and assigned them to the correct VLAN.

On S1 (A1, B1, C1):

int fa0/2

switchport mode access

switchport access vlan 10

no shutdown

exit

int fa0/3

switchport mode access

switchport access vlan 20

no shutdown

exit

int fa0/4

switchport mode access

switchport access vlan 30

no shutdown

exit

On S2 (A2, B2, C2):

int fa0/1

switchport mode access

switchport access vlan 10

no shutdown

exit

int fa0/2

switchport mode access

switchport access vlan 20

no shutdown

exit

int fa0/3

switchport mode access

switchport access vlan 30

no shutdown

exit

On S3 (D1, D2):

int fa0/1

switchport mode access

switchport access vlan 40

no shutdown

exit

int fa0/2

switchport mode access

switchport access vlan 40

no shutdown

exit

On S4 (E1, F1, G1):

int fa0/4

switchport mode access

switchport access vlan 50

no shutdown

exit

int fa0/2

switchport mode access

switchport access vlan 60

no shutdown

exit

int fa0/1

switchport mode access

switchport access vlan 70

no shutdown

exit

## Phase 3: Trunk Configuration & VLAN Pruning

We configured inter-switch and router uplinks to carry multiple VLANs, restricting them to only the necessary ones.

On S5 (To R1, S1, S2):

int fa0/3

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

no shutdown

exit

int fa0/1

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

no shutdown

exit

int fa0/5

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

no shutdown

exit

On S1 (To S5, S2):

int fa0/1

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

no shutdown

exit

int fa0/10

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

no shutdown

exit

On S2 (To S5, S1):

int fa0/5

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

no shutdown

exit

int fa0/10

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

no shutdown

exit

On S4 (To R3):

int fa0/3

switchport mode trunk

switchport trunk allowed vlan 50,60,70,99

no shutdown

exit

On S6 (To R2, S3 - Initial trunks before EtherChannel):

int fa0/3

switchport mode trunk

switchport trunk allowed vlan 40,99

no shutdown

exit

int range fa0/17-19

switchport mode trunk

switchport trunk allowed vlan 40,99

no shutdown

exit

On S3 (To S6 - Initial trunks):

int range fa0/17-19

switchport mode trunk

switchport trunk allowed vlan 40,99

no shutdown

exit

## Phase 4: Router-on-a-Stick (Inter-VLAN Routing)

We created sub-interfaces on the 2911 routers to serve as default gateways for each VLAN.

On R1 (WAN G0/0 to R3, LAN G0/1 to S5):

int g0/0

ip address 83.217.64.57 255.255.255.252

no shutdown

exit

int g0/1

no shutdown

exit

int g0/1.10

encapsulation dot1Q 10

ip address 83.217.64.1 255.255.255.248

exit

int g0/1.20

encapsulation dot1Q 20

ip address 83.217.64.9 255.255.255.248

exit

int g0/1.30

encapsulation dot1Q 30

ip address 83.217.64.17 255.255.255.248

exit

int g0/1.99

encapsulation dot1Q 99

ip address 83.217.64.97 255.255.255.240

exit

On R3 (WAN to R1/R2, LAN G0/2 to S4):

int g0/0

ip address 83.217.64.58 255.255.255.252

no shutdown

exit

int g0/1

ip address 83.217.64.61 255.255.255.252

no shutdown

exit

int g0/2

no shutdown

exit

int g0/2.50

encapsulation dot1Q 50

ip address 83.217.64.33 255.255.255.248

exit

int g0/2.60

encapsulation dot1Q 60

ip address 83.217.64.41 255.255.255.248

exit

int g0/2.70

encapsulation dot1Q 70

ip address 83.217.64.49 255.255.255.248

exit

int g0/2.99

encapsulation dot1Q 99

ip address 83.217.64.104 255.255.255.240

exit

On R2 (WAN to R3, LAN G0/0 to S6):

int g0/1

ip address 83.217.64.62 255.255.255.252

no shutdown

exit

int g0/0

no shutdown

exit

int g0/0.40

encapsulation dot1Q 40

ip address 83.217.64.25 255.255.255.248

exit

int g0/0.99

encapsulation dot1Q 99

ip address 83.217.64.105 255.255.255.240

exit

## Phase 5: EtherChannel (S6 ↔ S3)

We bundled the 3 parallel links into a single logical Port-Channel. Includes the troubleshooting fix where we applied trunk config directly to the Port-Channel.

On S6:

int port-channel 1

switchport mode trunk

switchport trunk allowed vlan 40,99

no shutdown

exit

int range fa0/17-19

channel-group 1 mode active

switchport mode trunk

switchport trunk allowed vlan 40,99

no shutdown

exit

On S3:

int port-channel 1

switchport mode trunk

switchport trunk allowed vlan 40,99

no shutdown

exit

int range fa0/17-19

channel-group 1 mode active

switchport mode trunk

switchport trunk allowed vlan 40,99

no shutdown

exit

## Phase 6: SVIs & Management Subnet Split

We assigned IPs to the switches. When the single subnet .96/28 failed due to connected route conflicts, we split it into .96/28 (Left), .128/28 (Middle), and .112/28 (Right).

Initial SVIs (Left Side - Unchanged):

On S5: int vlan 99 / ip add 83.217.64.98 255.255.255.240 / no shut / ip default-gateway 83.217.64.97

On S1: int vlan 99 / ip add 83.217.64.99 255.255.255.240 / no shut / ip default-gateway 83.217.64.97

On S2: int vlan 99 / ip add 83.217.64.100 255.255.255.240 / no shut / ip default-gateway 83.217.64.97

Middle Side (R3 & S4 - Changed to .128/28):

On R3: int g0/2.99 / ip address 83.217.64.129 255.255.255.240

On S4: int vlan 99 / ip add 83.217.64.130 255.255.255.240 / no shut / ip default-gateway 83.217.64.129

Right Side (R2, S6, S3 - Changed to .112/28):

On R2: int g0/0.99 / ip address 83.217.64.113 255.255.255.240

On S6: int vlan 99 / ip add 83.217.64.114 255.255.255.240 / no shut / ip default-gateway 83.217.64.113

On S3: int vlan 99 / ip add 83.217.64.115 255.255.255.240 / no shut / ip default-gateway 83.217.64.113

## Phase 7: Rapid PVST+ Path Control

We enabled Rapid PVST, set S5 as the Root, and manipulated costs to force VLAN 10/20 via S2, and VLAN 30 via S1. This includes the VLAN 20 tie-breaker fix using priority 40960.

On S5, S1, S2 (Enable RSTP):

spanning-tree mode rapid-pvst

On S5 (Root Bridge):

spanning-tree vlan 1 root primary

spanning-tree vlan 10 root primary

spanning-tree vlan 20 root primary

spanning-tree vlan 30 root primary

spanning-tree vlan 99 root primary

On S1 (Block direct path to S5 for VLAN 10 & 20 - Force via S2):

int fa0/1

spanning-tree vlan 10 cost 100

spanning-tree vlan 20 cost 200   ! (Increased to 200)

exit

On S1 (Tie-breaker fix for VLAN 20, after STP still preferred Fa0/1 due to lower MAC address):

spanning-tree vlan 20 priority 40960

On S2 (Block direct path to S5 for VLAN 30 - Force via S1):

int fa0/5

spanning-tree vlan 30 cost 100

exit

Extra link S1-S2 (Fa0/11) - configured as a separate trunk for redundancy. On S1 & S2:

int fa0/11

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

no shutdown

## Phase 8: Static Routing (End-to-End Connectivity)

We added manual routes so routers know how to reach remote subnets. Includes the additional WAN subnet routes we added during troubleshooting.

On R1:

ip route 83.217.64.24 255.255.255.248 83.217.64.58    ! VLAN 40

ip route 83.217.64.32 255.255.255.248 83.217.64.58    ! VLAN 50

ip route 83.217.64.40 255.255.255.248 83.217.64.58    ! VLAN 60

ip route 83.217.64.48 255.255.255.248 83.217.64.58    ! VLAN 70

ip route 83.217.64.112 255.255.255.240 83.217.64.58   ! Mgmt Right

ip route 83.217.64.128 255.255.255.240 83.217.64.58   ! Mgmt Middle

ip route 83.217.64.60 255.255.255.252 83.217.64.58    ! WAN R3-R2

On R3:

ip route 83.217.64.0 255.255.255.248 83.217.64.57     ! VLAN 10

ip route 83.217.64.8 255.255.255.248 83.217.64.57     ! VLAN 20

ip route 83.217.64.16 255.255.255.248 83.217.64.57    ! VLAN 30

ip route 83.217.64.96 255.255.255.240 83.217.64.57    ! Mgmt Left

ip route 83.217.64.24 255.255.255.248 83.217.64.62    ! VLAN 40

ip route 83.217.64.112 255.255.255.240 83.217.64.62   ! Mgmt Right

On R2:

ip route 83.217.64.0 255.255.255.248 83.217.64.61     ! VLAN 10

ip route 83.217.64.8 255.255.255.248 83.217.64.61     ! VLAN 20

ip route 83.217.64.16 255.255.255.248 83.217.64.61    ! VLAN 30

ip route 83.217.64.32 255.255.255.248 83.217.64.61    ! VLAN 50

ip route 83.217.64.40 255.255.255.248 83.217.64.61    ! VLAN 60

ip route 83.217.64.48 255.255.255.248 83.217.64.61    ! VLAN 70

ip route 83.217.64.96 255.255.255.240 83.217.64.61    ! Mgmt Left

ip route 83.217.64.128 255.255.255.240 83.217.64.61   ! Mgmt Middle

ip route 83.217.64.56 255.255.255.252 83.217.64.61    ! WAN R1-R3