4 routers

R0 , R1 , R2 , R3

2 switches 
SW0 , SW1

9 PCs 
PC0 ,PC1 ,PC2 ,PC3 , PC4 , PC5 , PC6 , PC7 , PC8, P

Connections :

PC0  SW0 on fa0/4 (vlan 11 )

PC1 SW0 on fa0/5 (vlan 11 )

PC2 SW0 fa0/6 (vlan 12 )

PC3 SW1 fa0/4 (vlan 12 )

PC4  SW1 on fa0/5 (vlan 13 )

PC5 SW0 on fa0/6 (vlan 14 )

PC6 SW0 fa0/7 (vlan 15 )

PC7 SW1 fa0/8 (vlan 16 )

PC8 (phiscally disconnectd) (vlan 16 )

| Source Device | Source Port | Destination Device | Destination Port | Link Type |  |
| --- | --- | --- | --- | --- | --- |
| Router0 | GigabitEthernet0/0 | SW0 | FastEthernet0/1 | Copper (Gig↔Fast) |  |
| Router0 | GigabitEthernet0/1 | SW1 | FastEthernet0/1 | Copper (Gig↔Fast) |  |
| Router1 | GigabitEthernet0/0 | SW0 | FastEthernet0/2 | Copper |  |
| Router1 | GigabitEthernet0/1 | SW1 | FastEthernet0/2 | Copper |  |
| Router2 | GigabitEthernet0/0 | Router3 | GigabitEthernet0/0 | Copper (Gig↔Gig) |  |
| Router0 | GigabitEthernet0/0 | SW0 | Fa0/1 | Copper |  |
| SW0 | FastEthernet0/3 | SW1 | FastEthernet0/3 | Copper (Redundant Trunk) |  |
| SW0 | FastEthernet0/4 | PC0 | FastEthernet0 | Copper |  |
| SW0 | FastEthernet0/5 | PC1 | FastEthernet0 | Copper |  |
| SW0 | FastEthernet0/6 | PC2 | FastEthernet0 | Copper |  |
| SW1 | FastEthernet0/4 | PC3 | FastEthernet0 | Copper |  |
| SW1 | FastEthernet0/5 | PC4 | FastEthernet0 | Copper |  |
| SW1 | FastEthernet0/6 | PC5 | FastEthernet0 | Copper |  |
| SW1 | FastEthernet0/7 | PC6 | FastEthernet0 | Copper |  |
| SW1 | FastEthernet0/8 | PC7 | FastEthernet0 | Copper |  |
| Router1 | Serial0/1/0 | Router2 | Serial0/1/0 | Serial (WAN) |  |
| Router2 | Gig0/0 | Router3 | Gig0/0 |  |  |

Networks :
between R1 and R2 : 20.20.20.0/24

Between R2 and R3 : 144.44.44.0/24

Look into the given topology and as a network engineer you have been asked to achieve the below

1 ++ you need to use this subnet and make subnetting so that each vlan is taking only the needed number of hosts and to make sure that no overlap between vlan will occur

Given subnet : 200.0.0.0/24

Vlan 11 : JU : 59 hosts

Vlan 14 : JUST : 58 hosts

Vlan 12 : HU : 28 hosts

Vlan 15 : MU : 13 hosts

Vlan 13 : BAU : 12 hosts

VLAN 16: ZU : 5 hosts

Write each vlan network address and subnet mask

Goal: Use 200.0.0.0/24 to create 6 subnets for the 6 VLANs. We use the formula 2n-2≥Hosts.

| VLAN | Name | Hosts | Formula | Mask | Network Address | Range | Gateway |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 11 | JU | 59 | 59+2=61 → 2^6=64 | /26 | 200.0.0.0/26 | .1 – .62 | .1 |
| 14 | JUST | 58 | 58+2=60 → 64 | /26 | 200.0.0.64/26 | .65 – .126 | .65 |
| 12 | HU | 28 | 28+2=30 → 2^5=32 | /27 | 200.0.0.128/27 | .129 – .158 | .129 |
| 15 | MU | 13 | 13+2=15 → 2^4=16 | /28 | 200.0.0.160/28 | .161 – .174 | .161 |
| 13 | BAU | 12 | 12+2=14 → 16 | /28 | 200.0.0.176/28 | .177 – .190 | .177 |
| 16 | ZU | 5 | 5+2=7 → 2^3=8 | /29 | 200.0.0.192/29 | .193 – .198 | .193 |

2 ++ write the first valid ip address for JU and ZU

JU : 200.0.0.1

ZU : 200.0.0.193

3 ++ you need to configure the needed vlans  and interfaces on SW0 and SW1

( vlan name also should be configured on switches)

4 ++ R0 & R1 are running with redundancy using HSRP protocol and R1 should be the active router for all vlans

5 ++ you need to specify the GW for each VLAN considering that it should be a valid ip address from the resulted networks after subnetting

6 ++ configure R0 and R1 as a DHCP server for VLAN 13 and VLAN 14

7 ++ use OSPF between R0,R1 ,R2 to make all pcs reach the internet but network 144.44.44.0/30 should not be advertised in OSPF config

** Do not change any pre-configuration on R2 and internet router should not have any ospf config

8 ++ you have been asked to make the interface on sw 1 that connecting PC7 only operational if PC7 is the connected one while if we try to connect PC8 on this interface , the interface should be in blocking state but if we connect PC7 back after 60 seconds the interface should be operational again without any human action

In a real Cisco IOS environment, we would enable automatic recovery with:

errdisable recovery cause psecure-violation

errdisable recovery interval 60

However, Packet Tracer’s 2960 switch does not support this command. Therefore, we demonstrate the manual recovery process. The concept is fully functional on production equipment

9 ++ use R2 as a PAT router with the below details

All pcs in VLAN11 , vlan 12 , vlan 13 should be translated to address from pool  ( 144.44.44.10 to144.44.44.11 )

All pcs in VLAN14 , vlan 15 , vlan 16 should be translated to address from pool  ( 144.44.44.20 to 144.44.44.22 )

10 ++ PC2 , PC3 should be able to telnet R0 and when doing this R0 should ask for username and password

11 ++ All pcs in vlan 16 should not be able to reach pcs in VLAN 12 but should be able to reach everyone else

12++ All other pc7 should reach each other