NETWORK DESIGN & IMPLEMENTATION

Multi-Site Enterprise Network Lab — Full Technical Documentation

VLAN Segmentation  ·  EtherChannel  ·  Router-on-a-Stick  ·  Static Routing  ·  DHCP  ·  ACLs

Prepared as a Cisco Packet Tracer Laboratory Exercise

Document Version 1.0

# 1. Lab Overview & Requirements

## 1.1 Objective

This document describes the design, configuration, and verification of a multi-site enterprise network built in Cisco Packet Tracer. The network demonstrates a range of advanced switching and routing technologies, including VLAN segmentation, EtherChannel link aggregation, Router-on-a-Stick inter-VLAN routing, static routing between sites, centralized DHCP services, and Access Control Lists (ACLs) for traffic filtering.

## 1.2 Topology Summary

The network is organized around a Layer 3 distribution switch (MLS1) that aggregates three access-layer switches (S1, S2, S3) serving VLANs 2–5, and two edge routers (R1 and R2) that each provide Router-on-a-Stick service for a separate access switch (S4 and S5) hosting VLANs 6–8 and 9–10 respectively. A centralized DHCP server, attached to R2, provides addressing for all VLANs except VLAN 9, which is intentionally isolated for both DHCP and general reachability testing.

## 1.3 Required Hardware

| Device Type | Model | Quantity | Hostname(s) |
| --- | --- | --- | --- |
| Layer 3 Switch | 3560-24PS | 1 | MLS1 |
| Layer 2 Switch | 2960 | 5 | S1, S2, S3, S4, S5 |
| Router | 2911 | 2 | R1, R2 |
| PC | Generic | 9 | PC2 – PC10 |
| Server | Generic | 1 | DHCP Server |

## 1.4 Physical Connections (Port Table)

| Device A | Port(s) | Connected To | Port(s) | Type |
| --- | --- | --- | --- | --- |
| MLS1 | Fa0/1–2 | S3 | Fa0/1–2 | Port-Channel (2 links) |
| MLS1 | G0/1 | R1 | G0/0 | Point-to-point /30 |
| S3 | Fa0/17–18 | S1 | Fa0/17–18 | Port-Channel (2 links) |
| S3 | Fa0/19–20 | S2 | Fa0/19–20 | 2 physical links (no Port-Channel) |
| S1 | Fa0/3–4 | S2 | Fa0/3–4 | Port-Channel (2 links) |
| S1 | Fa0/1 | PC2 | Fa0 | Access port |
| S1 | Fa0/2 | PC3 | Fa0 | Access port |
| S2 | Fa0/1 | PC4 | Fa0 | Access port |
| S2 | Fa0/2 | PC5 | Fa0 | Access port |
| S4 | Fa0/1 | PC6 | Fa0 | Access port |
| S4 | Fa0/2 | PC7 | Fa0 | Access port |
| S4 | Fa0/3 | PC8 | Fa0 | Access port |
| S4 | Fa0/24 | R1 | G0/2 | Trunk (Router-on-a-Stick) |
| S5 | Fa0/1 | PC9 | Fa0 | Access port |
| S5 | Fa0/2 | PC10 | Fa0 | Access port |
| S5 | Fa0/24 | R2 | G0/1 | Trunk (Router-on-a-Stick) |
| R1 | G0/0 | MLS1 | G0/1 | Point-to-point /30 |
| R1 | G0/1 | R2 | G0/0 | Point-to-point /30 |
| R2 | G0/2 | DHCP Server | Fa0 | Access (static IP) |

## 1.5 VLAN & IP Addressing (VLSM)

Addressing follows a simple mnemonic rule: VLAN X is assigned the network X0.0.0.0, which keeps the addressing scheme easy to read and audit.

| VLAN | Required Hosts | CIDR | Subnet Mask | Network Address | Usable Range |
| --- | --- | --- | --- | --- | --- |
| VLAN 2 | 10 | /28 | 255.255.255.240 | 20.0.0.0 | 20.0.0.1 – 20.0.0.14 |
| VLAN 3 | 192 | /24 | 255.255.255.0 | 30.0.0.0 | 30.0.0.1 – 30.0.0.254 |
| VLAN 4 | 200 | /24 | 255.255.255.0 | 40.0.0.0 | 40.0.0.1 – 40.0.0.254 |
| VLAN 5 | 50 | /26 | 255.255.255.192 | 50.0.0.0 | 50.0.0.1 – 50.0.0.62 |
| VLAN 6 | 70 | /25 | 255.255.255.128 | 60.0.0.0 | 60.0.0.1 – 60.0.0.126 |
| VLAN 7 | 67 | /25 | 255.255.255.128 | 70.0.0.0 | 70.0.0.1 – 70.0.0.126 |
| VLAN 8 | 63 | /25 | 255.255.255.128 | 80.0.0.0 | 80.0.0.1 – 80.0.0.126 |
| VLAN 9 | 123 | /25 | 255.255.255.128 | 90.0.0.0 | 90.0.0.1 – 90.0.0.126 |
| VLAN 10 | 67 | /25 | 255.255.255.128 | 100.0.0.0 | 100.0.0.1 – 100.0.0.126 |
| MLS1 ↔ R1 | 2 | /30 | 255.255.255.252 | 100.0.0.128 | 100.0.0.129 – 100.0.0.130 |
| R1 ↔ R2 | 2 | /30 | 255.255.255.252 | 101.0.0.0 | 101.0.0.1 – 101.0.0.2 |
| R2 ↔ DHCP Server | 254 | /24 | 255.255.255.0 | 200.0.0.0 | 200.0.0.1 – 200.0.0.254 |

## 1.6 Complete IP Addressing Table

| Device | Interface / VLAN | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| MLS1 | VLAN 2 | 20.0.0.1 | 255.255.255.240 | N/A |
| MLS1 | VLAN 3 | 30.0.0.1 | 255.255.255.0 | N/A |
| MLS1 | VLAN 4 | 40.0.0.1 | 255.255.255.0 | N/A |
| MLS1 | VLAN 5 | 50.0.0.1 | 255.255.255.192 | N/A |
| MLS1 | G0/1 | 100.0.0.129 | 255.255.255.252 | N/A |
| R1 | G0/0 | 100.0.0.130 | 255.255.255.252 | N/A |
| R1 | G0/1 | 101.0.0.1 | 255.255.255.252 | N/A |
| R1 | G0/2.6 | 60.0.0.1 | 255.255.255.128 | N/A |
| R1 | G0/2.7 | 70.0.0.1 | 255.255.255.128 | N/A |
| R1 | G0/2.8 | 80.0.0.1 | 255.255.255.128 | N/A |
| R2 | G0/0 | 101.0.0.2 | 255.255.255.252 | N/A |
| R2 | G0/1.9 | 90.0.0.1 | 255.255.255.128 | N/A |
| R2 | G0/1.10 | 100.0.0.1 | 255.255.255.128 | N/A |
| R2 | G0/2 | 200.0.0.1 | 255.255.255.0 | N/A |
| DHCP Server | Fa0 | 200.0.0.2 | 255.255.255.0 | 200.0.0.1 |
| PC2 | Fa0 | DHCP (20.0.0.3 – 14) | 255.255.255.240 | 20.0.0.1 |
| PC3 | Fa0 | DHCP (30.0.0.3 – 254) | 255.255.255.0 | 30.0.0.1 |
| PC4 | Fa0 | DHCP (40.0.0.3 – 254) | 255.255.255.0 | 40.0.0.1 |
| PC5 | Fa0 | DHCP (50.0.0.3 – 62) | 255.255.255.192 | 50.0.0.1 |
| PC6 | Fa0 | DHCP (60.0.0.3 – 126) | 255.255.255.128 | 60.0.0.1 |
| PC7 | Fa0 | DHCP (70.0.0.3 – 126) | 255.255.255.128 | 70.0.0.1 |
| PC8 | Fa0 | DHCP (80.0.0.3 – 126) | 255.255.255.128 | 80.0.0.1 |
| PC9 | Fa0 | 90.0.0.2 (static) | 255.255.255.128 | 90.0.0.1 |
| PC10 | Fa0 | DHCP (100.0.0.3 – 126) | 255.255.255.128 | 100.0.0.1 |

# 2. Configuration Phases (In Order)

## 2.1 Phase 1 — VLAN Creation & Access Ports

VLANs are created on each switch that requires local Layer 2 segmentation before any trunk or access-port configuration is applied.

### VLANs on MLS1, S3, S1, S2

vlan 2
 name VLAN_2
exit
vlan 3
 name VLAN_3
exit
vlan 4
 name VLAN_4
exit
vlan 5
 name VLAN_5
exit

### VLANs on S4

vlan 6
 name VLAN_6
exit
vlan 7
 name VLAN_7
exit
vlan 8
 name VLAN_8
exit

### VLANs on S5

vlan 9
 name VLAN_9
exit
vlan 10
 name VLAN_10
exit

### Access Ports — Example (S1: PC2, PC3)

interface fastEthernet 0/1
 switchport mode access
 switchport access vlan 2
 no shutdown
exit
interface fastEthernet 0/2
 switchport mode access
 switchport access vlan 3
 no shutdown
exit

The same pattern is repeated on the remaining access switches: S2 (PC4 → VLAN 4, PC5 → VLAN 5), S4 (PC6 → VLAN 6, PC7 → VLAN 7, PC8 → VLAN 8), and S5 (PC9 → VLAN 9, PC10 → VLAN 10).

## 2.2 Phase 2 — Trunks & EtherChannel

Troubleshooting Note —  EtherChannel VLAN Mask Mismatch. While bundling ports into a Port-Channel, the switch reported the error shown below.

%EC-5-CANNOT_BUNDLE2: Fa0/1 is not compatible with Po1 and will be suspended (vlan mask is different)

Cause: the physical ports carried a different VLAN configuration than the Port-Channel interface itself, so EtherChannel refused to bundle them.

Solution: remove the physical ports from the channel group, reset them to their default configuration, and rebuild the trunk configuration identically on both the Port-Channel interface and the member ports before re-adding them to the channel group.

### EtherChannel Configuration — MLS1 ↔ S3 (Po1)

On MLS1:

interface port-channel 1
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk allowed vlan 2,3,4,5
 no shutdown
exit
interface range fastEthernet 0/1-2
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk allowed vlan 2,3,4,5
 channel-group 1 mode active
 no shutdown
exit

On S3 (identical configuration):

interface port-channel 1
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk allowed vlan 2,3,4,5
 no shutdown
exit
interface range fastEthernet 0/1-2
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk allowed vlan 2,3,4,5
 channel-group 1 mode active
 no shutdown
exit

The same pattern (Port-Channel first, then matching physical-port configuration, then channel-group membership) is repeated for S3 ↔ S1 (Po2) and S1 ↔ S2 (Po1), each using the appropriate VLAN allowed list for that link.

### Trunk — S3 ↔ S2 (No Port-Channel)

interface range fastEthernet 0/19-20
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk allowed vlan 4,5
 no shutdown
exit

### Router-Switch Trunks

S4 (to R1):

interface fastEthernet 0/24
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk allowed vlan 6,7,8
 no shutdown
exit

S5 (to R2):

interface fastEthernet 0/24
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk allowed vlan 9,10
 no shutdown
exit

## 2.3 Phase 3 — SVIs & Router-on-a-Stick

### Switched Virtual Interfaces (SVIs) on MLS1

ip routing
interface vlan 2
 ip address 20.0.0.1 255.255.255.240
 no shutdown
exit
interface vlan 3
 ip address 30.0.0.1 255.255.255.0
 no shutdown
exit
interface vlan 4
 ip address 40.0.0.1 255.255.255.0
 no shutdown
exit
interface vlan 5
 ip address 50.0.0.1 255.255.255.192
 no shutdown
exit

### Router-on-a-Stick — R1 (VLANs 6, 7, 8)

interface gigabitEthernet 0/2
 no shutdown
exit
interface gigabitEthernet 0/2.6
 encapsulation dot1Q 6
 ip address 60.0.0.1 255.255.255.128
 no shutdown
exit
interface gigabitEthernet 0/2.7
 encapsulation dot1Q 7
 ip address 70.0.0.1 255.255.255.128
 no shutdown
exit
interface gigabitEthernet 0/2.8
 encapsulation dot1Q 8
 ip address 80.0.0.1 255.255.255.128
 no shutdown
exit

### Router-on-a-Stick — R2 (VLANs 9, 10)

interface gigabitEthernet 0/1
 no shutdown
exit
interface gigabitEthernet 0/1.9
 encapsulation dot1Q 9
 ip address 90.0.0.1 255.255.255.128
 no shutdown
exit
interface gigabitEthernet 0/1.10
 encapsulation dot1Q 10
 ip address 100.0.0.1 255.255.255.128
 no shutdown
exit

### Point-to-Point Links

! MLS1 G0/1
interface gigabitEthernet 0/1
 no switchport
 ip address 100.0.0.129 255.255.255.252
 no shutdown
exit
 
! R1 G0/0
interface gigabitEthernet 0/0
 ip address 100.0.0.130 255.255.255.252
 no shutdown
exit
 
! R1 G0/1
interface gigabitEthernet 0/1
 ip address 101.0.0.1 255.255.255.252
 no shutdown
exit
 
! R2 G0/0
interface gigabitEthernet 0/0
 ip address 101.0.0.2 255.255.255.252
 no shutdown
exit
 
! R2 G0/2 to DHCP Server
interface gigabitEthernet 0/2
 ip address 200.0.0.1 255.255.255.0
 no shutdown
exit

## 2.4 Phase 4 — Static Routing

### On MLS1

ip route 60.0.0.0 255.255.255.128 100.0.0.130
ip route 70.0.0.0 255.255.255.128 100.0.0.130
ip route 80.0.0.0 255.255.255.128 100.0.0.130
ip route 90.0.0.0 255.255.255.128 100.0.0.130
ip route 100.0.0.0 255.255.255.128 100.0.0.130
ip route 200.0.0.0 255.255.255.0 100.0.0.130
ip route 0.0.0.0 0.0.0.0 100.0.0.130   ! Optional default route

### On R1

ip route 20.0.0.0 255.255.255.240 100.0.0.129
ip route 30.0.0.0 255.255.255.0 100.0.0.129
ip route 40.0.0.0 255.255.255.0 100.0.0.129
ip route 50.0.0.0 255.255.255.192 100.0.0.129
ip route 90.0.0.0 255.255.255.128 101.0.0.2
ip route 100.0.0.0 255.255.255.128 101.0.0.2
ip route 200.0.0.0 255.255.255.0 101.0.0.2
ip route 0.0.0.0 0.0.0.0 101.0.0.2   ! Optional default route

### On R2

ip route 20.0.0.0 255.255.255.240 101.0.0.1
ip route 30.0.0.0 255.255.255.0 101.0.0.1
ip route 40.0.0.0 255.255.255.0 101.0.0.1
ip route 50.0.0.0 255.255.255.192 101.0.0.1
ip route 60.0.0.0 255.255.255.128 101.0.0.1
ip route 70.0.0.0 255.255.255.128 101.0.0.1
ip route 80.0.0.0 255.255.255.128 101.0.0.1
ip route 100.0.0.128 255.255.255.252 101.0.0.1
ip route 0.0.0.0 0.0.0.0 101.0.0.1   ! Optional default route

## 2.5 Phase 5 — DHCP Server & IP Helper

DHCP scopes are configured through the server's GUI, with the server itself addressed at 200.0.0.2. All VLANs are served except VLAN 9, which is deliberately excluded per the lab requirements.

| Pool Name | Default Gateway | DNS Server | Start IP | Subnet Mask | Max Users |
| --- | --- | --- | --- | --- | --- |
| VLAN2_Pool | 20.0.0.1 | 200.0.0.2 | 20.0.0.3 | 255.255.255.240 | 12 |
| VLAN3_Pool | 30.0.0.1 | 200.0.0.2 | 30.0.0.3 | 255.255.255.0 | 252 |
| VLAN4_Pool | 40.0.0.1 | 200.0.0.2 | 40.0.0.3 | 255.255.255.0 | 252 |
| VLAN5_Pool | 50.0.0.1 | 200.0.0.2 | 50.0.0.3 | 255.255.255.192 | 60 |
| VLAN6_Pool | 60.0.0.1 | 200.0.0.2 | 60.0.0.3 | 255.255.255.128 | 124 |
| VLAN7_Pool | 70.0.0.1 | 200.0.0.2 | 70.0.0.3 | 255.255.255.128 | 124 |
| VLAN8_Pool | 80.0.0.1 | 200.0.0.2 | 80.0.0.3 | 255.255.255.128 | 124 |
| VLAN10_Pool | 100.0.0.1 | 200.0.0.2 | 100.0.0.3 | 255.255.255.128 | 124 |

Note: no DHCP pool is created for VLAN 9, per the lab requirement.

### IP Helper-Address Configuration

On MLS1 (VLANs 2, 3, 4, 5):

interface vlan 2
 ip helper-address 200.0.0.2
exit
interface vlan 3
 ip helper-address 200.0.0.2
exit
interface vlan 4
 ip helper-address 200.0.0.2
exit
interface vlan 5
 ip helper-address 200.0.0.2
exit

On R1 (VLANs 6, 7, 8):

interface gigabitEthernet 0/2.6
 ip helper-address 200.0.0.2
exit
interface gigabitEthernet 0/2.7
 ip helper-address 200.0.0.2
exit
interface gigabitEthernet 0/2.8
 ip helper-address 200.0.0.2
exit

On R2 (VLAN 10 only — deliberately NOT applied to VLAN 9):

interface gigabitEthernet 0/1.10
 ip helper-address 200.0.0.2
exit

## 2.6 Phase 6 — Access Control Lists (ACLs)

### Requirement 1: VLAN 9 Cannot Reach the DHCP Server

On R2:

access-list 100 deny ip 90.0.0.0 0.0.0.127 host 200.0.0.2
access-list 100 permit ip any any
interface gigabitEthernet 0/1.9
 ip access-group 100 in
exit

### Requirement 2: VLAN 6 Cannot Communicate with VLAN 4

Troubleshooting Note —  ACL failed on first attempt. PC6 could still ping PC4 after the initial ACL was applied.

Initial (incorrect) application:

interface vlan 4
 ip access-group 101 in

Root cause: the ACL was applied inbound on VLAN 4, but traffic originating from PC6 (VLAN 6) enters MLS1 through the VLAN 6 SVI, is routed internally, and only then exits through the VLAN 4 SVI. An inbound ACL on VLAN 4 only inspects traffic arriving from locally attached VLAN 4 ports — it never sees traffic that has already been routed in from another VLAN.

Resolution —  the ACL must instead be applied outbound on the VLAN 6 SVI, so that it inspects traffic as it leaves toward other VLANs after routing.

Corrected application:

access-list 101 deny ip 60.0.0.0 0.0.0.127 40.0.0.0 0.0.0.255
access-list 101 permit ip any any
interface vlan 6
 ip access-group 101 out
exit

Verification: PC6 → PC4 traffic is blocked, while PC6 → PC5 traffic is still permitted, confirming the ACL is scoped correctly.

# 3. Final Verification & Testing

## 3.1 DHCP Tests

| PC | Expected Result | Status |
| --- | --- | --- |
| PC2 | DHCP IP in 20.0.0.3 – 14 | ✅ Pass |
| PC3 | DHCP IP in 30.0.0.3 – 254 | ✅ Pass |
| PC4 | DHCP IP in 40.0.0.3 – 254 | ✅ Pass |
| PC5 | DHCP IP in 50.0.0.3 – 62 | ✅ Pass |
| PC6 | DHCP IP in 60.0.0.3 – 126 | ✅ Pass |
| PC7 | DHCP IP in 70.0.0.3 – 126 | ✅ Pass |
| PC8 | DHCP IP in 80.0.0.3 – 126 | ✅ Pass |
| PC9 | Static IP: 90.0.0.2 (no DHCP) | ✅ Pass |
| PC10 | DHCP IP in 100.0.0.3 – 126 | ✅ Pass |

## 3.2 ACL Tests

| Source | Destination | Expected | Status |
| --- | --- | --- | --- |
| PC9 (VLAN 9) | DHCP Server (200.0.0.2) | Blocked | ✅ Pass |
| PC10 (VLAN 10) | DHCP Server (200.0.0.2) | Allowed | ✅ Pass |
| PC6 (VLAN 6) | PC4 (VLAN 4) | Blocked | ✅ Pass |
| PC6 (VLAN 6) | PC5 (VLAN 5) | Allowed | ✅ Pass |
| PC7 (VLAN 7) | PC4 (VLAN 4) | Allowed | ✅ Pass |

## 3.3 End-to-End Connectivity Tests

| Source | Destination | Expected | Status |
| --- | --- | --- | --- |
| PC2 (VLAN 2) | PC10 (VLAN 10) | Reply | ✅ Pass |
| PC4 (VLAN 4) | PC8 (VLAN 8) | Reply | ✅ Pass |
| PC6 (VLAN 6) | PC9 (VLAN 9) | Reply | ✅ Pass |
| PC9 (VLAN 9) | DHCP Server | Blocked by ACL | ✅ Pass |

## 3.4 EtherChannel Verification

Command:

show etherchannel summary

output:

Group  Port-channel  Protocol    Ports
------+-------------+-----------+----------------------------------------
1      Po1(SU)           LACP    Fa0/1(P) Fa0/2(P)

## 3.5 Trunk Verification

Command:

show interface trunk

output:

Port        Mode         Encapsulation  Status        Native vlan
Po1         on           802.1q         trunking      1

# 4. Troubleshooting Log Summary

The table below consolidates the issues encountered during implementation, together with their root causes and applied resolutions.

| Issue | Symptom | Cause | Solution |
| --- | --- | --- | --- |
| EtherChannel VLAN mask mismatch | %EC-5-CANNOT_BUNDLE2 error | Physical ports had a different VLAN allowed list than the Port-Channel interface. | Cleared the ports, configured the Port-Channel interface first, then added the physical ports with identical trunk settings. |
| ACL not blocking VLAN 6 → VLAN 4 | PC6 could still ping PC4 | ACL was applied inbound on VLAN 4, which does not inspect traffic already routed in from another VLAN. | Moved the ACL to the VLAN 6 SVI and applied it in the outbound direction. |

# 5. Conclusion

The network was successfully designed, configured, and verified against all stated lab requirements. The following capabilities were implemented and confirmed through testing:

- Variable Length Subnet Masking (VLSM) applied consistently across all VLANs and point-to-point links

- VLAN segmentation, 802.1Q trunking, and EtherChannel link aggregation

- Switched Virtual Interfaces (SVIs) and Router-on-a-Stick inter-VLAN routing

- Static routing across all three routing devices (MLS1, R1, R2)

- Centralized DHCP services via IP helper-address, excluding VLAN 9 by design

- Access Control Lists enforcing both VLAN 9 → DHCP Server isolation and VLAN 6 → VLAN 4 restriction

All verification tests — DHCP assignment, ACL enforcement, and end-to-end connectivity — passed as expected, confirming the network meets its design objectives.

📘 Version 2 Updates – Documentation Section (Corrected)

1. Overview of Updates

Version 2 of the lab introduced the following enhancements to the existing network design:

| Feature | Change |
| --- | --- |
| Dynamic Routing | Replaced static routing between R1 and R2 with OSPF (Open Shortest Path First) using specific network statements to enable dynamic route exchange. |
| PortFast | Enabled on all end-device access ports to accelerate STP convergence. |
| Port Security | Implemented on S4 access ports to restrict MAC address learning and enforce security policies. |

2. Detailed Configuration Changes

2.1 OSPF Implementation (Specific Networks)

Goal: Replace static routing between R1 and R2 with OSPF for dynamic route advertisement.

Reasoning: OSPF reduces manual configuration overhead and automatically adapts to network changes, making the network more scalable and resilient.

Configuration Performed:

- On R1 (Specific Networks):

router ospf 1

network 100.0.0.128 0.0.0.3 area 0   ! Link to MLS1

network 101.0.0.0 0.0.0.3 area 0     ! Link to R2

network 60.0.0.0 0.0.0.127 area 0    ! VLAN 6

network 70.0.0.0 0.0.0.127 area 0    ! VLAN 7

network 80.0.0.0 0.0.0.127 area 0    ! VLAN 8

redistribute static subnets

- On R2 (Specific Networks):

router ospf 1

network 101.0.0.0 0.0.0.3 area 0     ! Link to R1

network 90.0.0.0 0.0.0.127 area 0    ! VLAN 9

network 100.0.0.0 0.0.0.127 area 0   ! VLAN 10

network 200.0.0.0 0.0.0.255 area 0   ! DHCP Server network

Why Specific Networks?

- More secure and controlled OSPF activation.

- Prevents OSPF from accidentally running on unintended interfaces.

- Follows industry best practices for production environments.

Important: Static routes between MLS1 and R1 were retained as per the lab requirements.

2.2 PortFast Configuration

Goal: Enable PortFast on all switch ports connected to end devices to reduce STP convergence time from 50 seconds to immediate forwarding.

Reasoning: PortFast prevents DHCP timeouts and speeds up PC boot-up connectivity.

Configuration Applied (Example):

text

interface range fastEthernet 0/1-2

spanning-tree portfast

no shutdown

Devices Configured: S1, S2, S4, S5 (all access ports).

2.3 Port Security Configuration (S4)

Goal: Restrict unauthorized MAC address access on S4's access ports to improve network security.

| Port | PC | Security Settings |
| --- | --- | --- |
| Fa0/1 | PC6 | Maximum MACs = 2, Violation = Restrict |
| Fa0/2 | PC7 | Sticky MAC, Violation = Restrict |
| Fa0/3 | PC8 | Sticky MAC, Violation = Shutdown |

Verification Commands:

text

show port-security interface fastEthernet 0/1

show port-security address

3. Problem Identified After OSPF Implementation

Symptom:

- PC2 (VLAN 2) was unable to ping the DHCP Server (200.0.0.2).

- The tracert command showed successful hops up to R1 (100.0.0.130), but failed at R2.

- The ICMP request successfully reached the DHCP Server, but the reply was dropped at R2.

Root Cause Analysis:

| Step | Device | Issue |
| --- | --- | --- |
| 1 | PC2 → DHCP Server | ✅ Packet forwarded correctly |
| 2 | DHCP Server → PC2 | ✅ Reply sent to default gateway (R2) |
| 3 | R2 | ❌ R2 had no route to 20.0.0.0/28 (VLAN 2) |
| 4 | R2 | ❌ R2 dropped the packet and sent "Host Unreachable" |

Why R2 Had No Route:

- R1 had static routes for VLANs 2,3,4,5 (via MLS1).

- OSPF was enabled on R1 and R2 using specific network statements, but static routes were not redistributed into OSPF.

- As a result, R2 never learned about VLANs 2,3,4,5.

4. Solution Implemented

Fix Applied on R1:

text

router ospf 1

redistribute static subnets

What This Does:

- Advertises all static routes on R1 (including VLANs 2,3,4,5) into OSPF.

- R2 now learns these routes as O E2 (OSPF External Type 2) entries.

Why This Was Necessary:

- OSPF only advertises networks that are either:

  - Directly connected and included in a network statement, OR

  - Redistributed from another routing source (static, connected, etc.).

- Since VLANs 2,3,4,5 were learned via static routes on R1, they had to be explicitly redistributed into OSPF.