CCNA PACKET TRACER LAB

Full Network Design & Configuration Documentation

Covers VLANs, Trunking, EtherChannel, VTP, Spanning Tree, HSRP, DHCP,
OSPF Multi-Area Routing, NAT, ACLs, and Telnet Access Control

Prepared for: CCNA Certification Practice

Document Date: August 5, 2026

# Table of Contents

1. Network Topology Overview	3

1.1 Device Inventory	3

1.2 Physical Connections (Key Links)	3

1.3 VLANs & Subnets	4

2. Configuration Phases by Side	5

Phase 0 – Initial Device Setup	5

Phase 1 – Right Side (VLANs 40 & 70, HSRP, DHCP)	5

Phase 2 – Middle Side (VLANs 80, 90, STP, EtherChannel, NAT)	9

Phase 3 – Left Side (VLANs 50, 65, Logical L3 Link)	12

Phase 4 – OSPF Configuration (All Routers & SW7)	14

Phase 5 – ACLs & Telnet	16

3. Troubleshooting & Fixes	17

3.1 HSRP Flapping on VLAN40	17

3.2 HSRP Roles Reversed	17

3.3 R3 Could Not Ping R4 (209.1.1.2)	17

3.4 R3 Fa0/1 and R2 Fa0/0 Missing IPs	17

3.5 SW7 ↔ R6 OSPF Not Forming	17

3.6 ACL on R1 Not Effective for PC1	18

3.7 show access-list Commands Not Working	18

4. Final Verification Summary	19

5. Final Commands to Save Configurations	19

# 1. Network Topology Overview

## 1.1 Device Inventory

The topology consists of six Cisco 2811 routers, six Layer 2 access switches, and one Layer 3 (multilayer) switch, organized across three OSPF areas.

| Device | Model | Interfaces | OSPF Area |
| --- | --- | --- | --- |
| R1 | 2811 | Fa0/0, Fa0/1 | Area 1 |
| R2 | 2811 | Fa0/0, Fa0/1 | Area 1 |
| R3 | 2811 | Fa0/0, Fa0/1, Fa1/0, Fa1/1 | Area 0 |
| R4 | 2811 | Fa0/0, Fa0/1, Fa1/0 | Area 0 |
| R5 | 2811 | Fa0/0, Fa0/1 | Area 2 |
| R6 | 2811 | Fa0/0, Fa0/1 | Area 2 |
| SW1 | 2960 | L2 | – |
| SW2 | 2960 | L2 | – |
| SW3 | 2960 | L2 | – |
| SW4 | 2960 | L2 | – |
| SW5 | 2960 | L2 | – |
| SW6 | 2960 | L2 | – |
| SW7 | 3560 | L3 (MLS) | Area 2 |

## 1.2 Physical Connections (Key Links)

| Link | Subnet |
| --- | --- |
| R1 Fa0/1 – R3 Fa1/0 | 200.22.22.0/30 |
| R2 Fa0/0 – R3 Fa0/1 | 210.3.3.0/30 |
| R3 Fa0/0 – R4 Fa0/1 | 209.1.1.0/29 |
| R4 Fa1/0 – R5 Fa0/0 | 167.67.67.0/30 |
| R4 Fa0/0 – R6 Fa0/0 | 179.12.15.0/30 |
| R6 Fa0/1 – SW7 Fa0/1 (Logical L3 via VLAN 10) | 10.10.10.0/30 |

## 1.3 VLANs & Subnets

| VLAN ID | Name | Associated Switches | End Devices | Subnet |
| --- | --- | --- | --- | --- |
| 40 | VLAN40 | SW1, SW2 | PC1 (Fa0/6 SW2) | 40.40.40.0/29 |
| 70 | VLAN70 | SW1, SW2 | PC2 (Fa0/5), PC3 (Fa0/4 SW2) | 70.70.70.0/29 |
| 80 | VLAN80 | SW3, SW4, SW5 | PC4 (Fa0/3 SW5) | 80.80.80.0/29 |
| 90 | VLAN90 | SW3, SW6 | PC6 (Fa0/4 SW6) | 90.90.90.0/29 |
| 50 | VLAN50 | SW7 | PC8 (Fa0/2 SW7) | 50.50.50.0/29 |
| 65 | VLAN65 | SW7 | PC7 (Fa0/3 SW7) | 65.65.65.0/29 |

# 2. Configuration Phases by Side

## Phase 0 – Initial Device Setup

- Set hostnames on all devices (no passwords per request).

- Example: hostname R1, hostname SW1, etc.

## Phase 1 – Right Side (VLANs 40 & 70, HSRP, DHCP)

1.1 SW1 (VTP Server)

vtp domain CCNA vtp password CCNA vtp version 2 vtp mode server ! vlan 40 name VLAN40 vlan 70 name VLAN70 vlan 57 name Native ! interface range fa0/3-5 channel-group 1 mode active ! interface port-channel 1 switchport trunk encapsulation dot1q switchport mode trunk switchport trunk native vlan 57 switchport trunk allowed vlan 1,40,70 ! interface fa0/1 switchport trunk encapsulation dot1q switchport mode trunk switchport trunk native vlan 57 switchport trunk allowed vlan 1,40,70 ! interface fa0/2 switchport trunk encapsulation dot1q switchport mode trunk switchport trunk native vlan 57 switchport trunk allowed vlan 1,40,70

1.2 SW2 (VTP Client)

vtp domain CCNA vtp password CCNA vtp version 2 vtp mode client ! interface range fa0/1-3 channel-group 1 mode active ! interface port-channel 1 switchport trunk encapsulation dot1q switchport mode trunk switchport trunk native vlan 57 switchport trunk allowed vlan 1,40,70 ! interface fa0/4 switchport mode access switchport access vlan 70 spanning-tree portfast interface fa0/5 switchport mode access switchport access vlan 70 spanning-tree portfast interface fa0/6 switchport mode access switchport access vlan 40 spanning-tree portfast

1.3 R1 (Subinterfaces, HSRP, DHCP)

interface fa0/0 no shutdown ! interface fa0/0.40 encapsulation dot1Q 40 ip address 40.40.40.2 255.255.255.248 standby 40 ip 40.40.40.1 standby 40 priority 100 standby 40 timers 3 10 ! interface fa0/0.70 encapsulation dot1Q 70 ip address 70.70.70.2 255.255.255.248 standby 70 ip 70.70.70.1 standby 70 priority 150 standby 70 timers 3 10 ! ip dhcp excluded-address 40.40.40.1 40.40.40.3 ip dhcp excluded-address 70.70.70.1 70.70.70.3 ! ip dhcp pool VLAN40 network 40.40.40.0 255.255.255.248 default-router 40.40.40.1 ! ip dhcp pool VLAN70 network 70.70.70.0 255.255.255.248 default-router 70.70.70.1 ! interface fa0/1 ip address 200.22.22.2 255.255.255.252 no shutdown

1.4 R2 (Subinterfaces, HSRP, DHCP)

interface fa0/1 no shutdown ! interface fa0/1.40 encapsulation dot1Q 40 ip address 40.40.40.3 255.255.255.248 standby 40 ip 40.40.40.1 standby 40 priority 150 standby 40 timers 3 10 ! interface fa0/1.70 encapsulation dot1Q 70 ip address 70.70.70.3 255.255.255.248 standby 70 ip 70.70.70.1 standby 70 priority 100 standby 70 timers 3 10 ! ip dhcp excluded-address 40.40.40.1 40.40.40.3 ip dhcp excluded-address 70.70.70.1 70.70.70.3 ! ip dhcp pool VLAN40 network 40.40.40.0 255.255.255.248 default-router 40.40.40.1 ! ip dhcp pool VLAN70 network 70.70.70.0 255.255.255.248 default-router 70.70.70.1 ! interface fa0/0 ip address 210.3.3.2 255.255.255.252 no shutdown

HSRP Final Roles (after troubleshooting) R1 is Active for VLAN70 and Standby for VLAN40. R2 is Active for VLAN40 and Standby for VLAN70. Preempt is disabled on both routers to lock these roles.

## Phase 2 – Middle Side (VLANs 80, 90, STP, EtherChannel, NAT)

2.1 SW3 (VTP Server, Root Bridge)

vtp domain CCNA1 vtp password CCNA1 vtp version 2 vtp mode server ! vlan 80 name VLAN80 vlan 90 name VLAN90 ! spanning-tree vlan 1 root primary spanning-tree vlan 80 root primary spanning-tree vlan 90 root primary ! interface range fa0/2, fa0/4 channel-group 1 mode active ! interface port-channel 1 switchport trunk encapsulation dot1q switchport mode trunk ! interface fa0/3 switchport trunk encapsulation dot1q switchport mode trunk ! interface fa0/1 switchport trunk encapsulation dot1q switchport mode trunk

2.2 SW4 (VTP Client)

vtp domain CCNA1 vtp password CCNA1 vtp version 2 vtp mode client ! interface fa0/1 switchport trunk encapsulation dot1q switchport mode trunk interface fa0/2 switchport trunk encapsulation dot1q switchport mode trunk

2.3 SW5 (VTP Transparent)

vtp domain CCNA1 vtp password CCNA1 vtp version 2 vtp mode transparent ! vlan 80 name VLAN80 ! interface fa0/2 switchport trunk encapsulation dot1q switchport mode trunk ! interface fa0/3 switchport mode access switchport access vlan 80 spanning-tree portfast

2.4 SW6 (VTP Client, Secondary Root)

vtp domain CCNA1 vtp password CCNA1 vtp version 2 vtp mode client ! spanning-tree vlan 1 root secondary spanning-tree vlan 90 root secondary ! interface range fa0/1, fa0/3 channel-group 1 mode active ! interface port-channel 1 switchport trunk encapsulation dot1q switchport mode trunk ! interface fa0/2 switchport trunk encapsulation dot1q switchport mode trunk ! interface fa0/4 switchport mode access switchport access vlan 90 spanning-tree portfast

2.5 R3 (NAT, OSPF)

interface fa0/0 ip address 209.1.1.3 255.255.255.248 no shutdown ip nat outside ! interface fa1/1 no shutdown ! interface fa1/1.80 encapsulation dot1Q 80 ip address 80.80.80.1 255.255.255.248 ip nat inside ! interface fa1/1.90 encapsulation dot1Q 90 ip address 90.90.90.1 255.255.255.248 ip nat inside ! interface fa1/0 ip address 200.22.22.1 255.255.255.252 no shutdown ! interface fa0/1 ip address 210.3.3.1 255.255.255.252 no shutdown ! ip nat inside source static 90.90.90.2 209.1.1.1

2.6 R4 (Routing, OSPF)

interface fa0/1 ip address 209.1.1.2 255.255.255.248 no shutdown ! interface fa1/0 ip address 167.67.67.1 255.255.255.252 no shutdown ! interface fa0/0 ip address 179.12.15.1 255.255.255.252 no shutdown

## Phase 3 – Left Side (VLANs 50, 65, Logical L3 Link)

3.1 SW7 (Layer 3 Switch)

ip routing ! vlan 10 name Link_to_R6 vlan 50 name VLAN50 vlan 65 name VLAN65 ! interface vlan 10 ip address 10.10.10.2 255.255.255.252 no shutdown ! interface vlan 50 ip address 50.50.50.1 255.255.255.248 no shutdown ! interface vlan 65 ip address 65.65.65.1 255.255.255.248 no shutdown ! interface fa0/1 switchport mode trunk switchport trunk allowed vlan 10 no shutdown ! interface fa0/2 switchport mode access switchport access vlan 50 spanning-tree portfast no shutdown ! interface fa0/3 switchport mode access switchport access vlan 65 spanning-tree portfast no shutdown

Note: Fa0/1 was initially set to access mode; it was changed to trunk mode to carry VLAN 10 tagging to R6.

3.2 R6

interface fa0/0 ip address 179.12.15.2 255.255.255.252 no shutdown ! interface fa0/1 no shutdown ! interface fa0/1.10 encapsulation dot1Q 10 ip address 10.10.10.1 255.255.255.252

3.3 R5

interface fa0/0 ip address 167.67.67.2 255.255.255.252 no shutdown ! interface loopback 0 ip address 1.1.1.1 255.255.255.255

## Phase 4 – OSPF Configuration (All Routers & SW7)

R1 (Area 1)

router ospf 1 router-id 1.1.1.1 network 40.40.40.0 0.0.0.7 area 1 network 70.70.70.0 0.0.0.7 area 1 network 200.22.22.0 0.0.0.3 area 1

R2 (Area 1)

router ospf 1 router-id 2.2.2.2 network 40.40.40.0 0.0.0.7 area 1 network 70.70.70.0 0.0.0.7 area 1 network 210.3.3.0 0.0.0.3 area 1

R3 (Area 0 + Area 1)

router ospf 1 router-id 3.3.3.3 network 80.80.80.0 0.0.0.7 area 0 network 90.90.90.0 0.0.0.7 area 0 network 209.1.1.0 0.0.0.7 area 0 network 200.22.22.0 0.0.0.3 area 1 network 210.3.3.0 0.0.0.3 area 1

R4 (Area 0 + Area 2)

router ospf 1 router-id 4.4.4.4 network 209.1.1.0 0.0.0.7 area 0 network 167.67.67.0 0.0.0.3 area 2 network 179.12.15.0 0.0.0.3 area 2

R5 (Area 2)

router ospf 1 router-id 5.5.5.5 network 167.67.67.0 0.0.0.3 area 2 network 1.1.1.1 0.0.0.0 area 2

R6 (Area 2)

router ospf 1 router-id 6.6.6.6 network 179.12.15.0 0.0.0.3 area 2 network 10.10.10.0 0.0.0.3 area 2

SW7 (Area 2)

router ospf 1 router-id 7.7.7.7 network 10.10.10.0 0.0.0.3 area 2 network 50.50.50.0 0.0.0.7 area 2 network 65.65.65.0 0.0.0.7 area 2

## Phase 5 – ACLs & Telnet

5.1 ACL for PC1 (only ping 1.1.1.1) – applied on R2 (Active HSRP router for VLAN40)

no access-list 100 access-list 100 permit icmp 40.40.40.0 0.0.0.7 host 1.1.1.1 access-list 100 deny icmp 40.40.40.0 0.0.0.7 any access-list 100 permit ip any any interface fa0/1.40 ip access-group 100 in

5.2 ACL for PC7 (cannot ping PC8) – applied on SW7 VLAN65

access-list 110 deny icmp host 65.65.65.2 host 50.50.50.2 echo access-list 110 permit ip any any interface vlan 65 ip access-group 110 in

5.3 Telnet on SW7 (only from PC8)

access-list 1 permit host 50.50.50.2 access-list 1 deny any line vty 0 4 password cisco login access-class 1 in

# 3. Troubleshooting & Fixes

## 3.1 HSRP Flapping on VLAN40

Issue: R1 repeatedly cycled through Speak → Standby → Active states.

Fix: Explicit HSRP timers (standby 40 timers 3 10) were added on both routers, and preempt was disabled to lock roles after failover.

## 3.2 HSRP Roles Reversed

Issue: R1 was Active for VLAN40 (it should have been Standby), while R2 was Standby.

Fix: Preempt was temporarily enabled on R2 for VLAN40 to force a takeover, then disabled again to lock the correct state. Priorities were already set correctly (R2 priority 150).

## 3.3 R3 Could Not Ping R4 (209.1.1.2)

Issue: Pings from R3 to 209.1.1.2 failed even though R4 was replying.

Fix: R3's IP on Fa0/0 was changed from .1 to .3 to avoid a conflict with the NAT global IP. A static ARP entry was also attempted for R4's MAC address; this approach did not resolve the issue directly — bouncing the interface and correcting the IP addressing ultimately allowed ARP to resolve normally.

arp 209.1.1.2 00d0.58dd.2402 arpa

## 3.4 R3 Fa0/1 and R2 Fa0/0 Missing IPs

Issue: R1 and R2 did not have their links to R3 configured.

Fix: IP addresses were added to R1 Fa0/1 (200.22.22.2) and R2 Fa0/0 (210.3.3.2), and both interfaces were enabled with no shutdown.

## 3.5 SW7 ↔ R6 OSPF Not Forming

Issue: No OSPF adjacency formed, and SW7 could not ping 10.10.10.1.

Root cause: SW7's Fa0/1 interface was in access mode (untagged), while R6 expected VLAN 10 tagged frames.

Fix: SW7 Fa0/1 was changed to trunk mode with VLAN 10 allowed.

interface fa0/1 switchport mode trunk switchport trunk allowed vlan 10

## 3.6 ACL on R1 Not Effective for PC1

Issue: PC1 was able to ping every destination despite ACL 100 being applied on R1.

Root cause: R2 was the Active HSRP router for VLAN40, so traffic was actually traversing R2, not R1.

Fix: The same ACL was applied to R2's Fa0/1.40 interface (the active gateway), and the ACL order was corrected — permit to 1.1.1.1 first, then deny all other ICMP.

## 3.7 show access-list Commands Not Working

Issue: Packet Tracer sometimes fails to display ACLs applied via show ip interface.

Fix: The following command was used instead to verify ACL application:

show running-config ¦ section interface Vlan65

# 4. Final Verification Summary

The table below summarizes the final connectivity and service tests performed across the topology.

| Test | Result |
| --- | --- |
| PC1 ping 1.1.1.1 | Success |
| PC1 ping any other PC | Failure |
| PC2, PC3 ping any other PC | Success |
| PC4, PC6 ping any other PC | Success |
| PC7 ping PC8 | Failure |
| PC7 ping any other PC | Success |
| PC8 telnet to SW7 (50.50.50.1) | Success, login cisco |
| PC7 telnet to SW7 | Failure |
| PC6 ping 1.1.1.1 | Success (NAT) |
| show ip nat translations on R3 | Shows translations for PC6 |

All devices have full OSPF adjacencies and complete routing tables, confirming that multi-area OSPF routing is functioning as designed. The failures observed for PC1 and PC7 are expected outcomes of the ACLs configured in Section 2.5 and are not defects.

# 5. Final Commands to Save Configurations

The following command was issued on every device to save the running configuration to NVRAM:

write memory

Scope Confirmation This lab satisfies all CCNA-level requirements including VLANs, trunking, EtherChannel, VTP, Spanning Tree, HSRP, DHCP, OSPF multi-area routing, NAT, ACLs, and Telnet access control.

— End of Documentation —